Assignment:- 2

Karnaugh Maps (K-maps)

- A Karnaugh map is a graphical method for simplifying Boolean algebra expressions.

- It organizes truth-table values into a grid so adjacent cells differ by only one variable (Gray code ordering).

- Goal: minimize logic (fewer gates) by grouping 1’s (for SOP) or 0’s (for POS) into powers of two.

### Types of K-maps-

### 1. 2 variable

### 2. 3 variable

### 3.4 variable

3. Rules / Steps for Using K-maps

- Draw the map

  - Choose 2, 3, or 4 variable grid.

  - Label rows/columns in Gray code order so only one bit changes between neighbors.

- Fill in the map

  - For Sum-of-Products (SOP): place 1’s where the function outputs 1.

  - For Product-of-Sums (POS): place 0’s where the function outputs 0.

- Group adjacent cells

  - Groups must be rectangles with sizes = powers of two: 1, 2, 4, 8…

  - Valid shapes: 1×1, 1×2, 2×1, 2×2, 4×1, etc.

  - Wrap-around edges are adjacent (top–bottom and left–right touch).

- Make the largest group possible always while grouping

  - Each group should cover the maximum number of 1’s (or 0’s) to eliminate the most variables.

- Simplify

  - For each group, write the product (for SOP) of variables that do not change within that group.

  - Combine all group terms with OR (for SOP) or AND (for POS).

- Check for coverage

  - Ensure every 1 (or 0) is covered at least once.

  - Essential groups (covering unique 1’s) must be included.

- Don’t care condition

- In this condition we do not care about the output for the corresponding cell and mark it with an ‘X’ sign.

- We can include this X in our grouping as per need

Quine–McCluskey Method (Tabular Method)

The Quine–McCluskey method is a step-by-step, tabular way to simplify Boolean expressions. It is especially useful when the number of variables is too large for a Karnaugh Map.

Why It Is Needed

- Digital circuits use Boolean logic.

- Simplifying expressions reduces the number of logic gates.

- This lowers cost, size, and delay of the circuit.

Key Terms

- Minterm: An AND term where the function is 1 for a specific input combination.

- Prime Implicant: A group of minterms combined together that cannot be reduced further.

- Essential Prime Implicant: A prime implicant that covers at least one minterm not covered by any other.

Steps of the Method

- List the minterms where the output is 1.

- Group minterms by the number of 1s in their binary form.

- Combine adjacent groups that differ by exactly one bit → replace the differing bit with a dash (–).

- Repeat combining until no further merging is possible.

  - Terms that cannot be combined further = Prime Implicants.

- Make a Prime Implicant Chart: rows = prime implicants, columns = minterms.

- Find Essential Prime Implicants: look for columns with only one “X”.

- Cover remaining minterms by selecting the fewest extra prime implicants.

- The final set gives the minimal Sum of Products (SOP) expression.

Example (Outline)

For
F(A,B,C,D) = Σm(0,1,2,5,6,7,8,9,10,14)

- Step 1: Convert minterms to binary and group them.

- Step 2–3: Combine until no more reduction is possible.

- Step 4: Identify prime implicants.

- Step 5–7: Select essentials and cover remaining terms.

- Step 8: Write the simplified Boolean function.

Advantages

- Works for any number of variables.

- Always gives the minimum expression.

Disadvantages

- Becomes lengthy by hand if variables > 5–6.

- Better handled by computer programs for large cases.

Applications

- Digital circuit design.

- Logic minimization in compilers and CAD tools.

- Teaching and learning Boolean simplification.

Top of Form

Bottom of Form