Assignment:- 3

Verification of Logic Gates and Boolean Algebra

The following are the logic gates and their outputs-

AND gate:-

OR gate:-

NAND gate:-

NOR gate:-

Laws for Boolean algebra:-

(a) Associative Law:-

- States that the grouping of variables does not affect the result.

- For AND (•):

(A•B)•C=A•(B•C)(A•B)•C=A•(B•C)(A•B)•C=A•(B•C)

- For OR (+):

(A+B)+C=A+(B+C)(A+B)+C=A+(B+\C)(A+B)+C=

A+(B+C)

(b) Distributive Law:-

- States that multiplication can be distributed over addition and vice versa.

- A•(B+C)=(A•B)+(A•C)A•(B+C)=(A•B)+(A•C)A•(B+C)=(A•B)+(A•C)

- A+(B•C)=(A+B)•(A+C)A+(B•C)=(A+B)•(A+C)A+

(B•C)=(A+B)•(A+C)

(c) Absorption Law:-

- Simplifies expressions by "absorbing" terms.

- A+(A•B)=AA + (A • B) = AA+(A•B)=A

- A•(A+B)=AA • (A + B) = AA•(A+B)=A

De Morgan’s Theorem:-

Used to simplify Boolean expressions involving NOT operations.

- (A•B)’=A’+B’(A • B)’ = A’ + B’(A•B)’=A’+B’

(Complement of AND = OR of complements)

- (A+B)’=A’•B’(A + B)’ = A’ • B’(A+B)’=A’•B’

(Complement of OR = AND of complements)

Principle of Duality:-

- Every Boolean equation remains valid if we interchange + and • and replace 0 with 1 and 1 with 0.

- Example:

  - Original: A+0=AA + 0 = AA+0=A

  - Dual: A•1=AA • 1 = AA•1=A

Minterm and SOP (Sum of Products):-

Minterm:

- A product (AND) term where each variable appears once (either complemented or uncomplemented).

- Example for 2 variables:

  - M0 = A’B’

  - M1 = A’B

  - M2 = AB’

  - M3 = AB

Sum of Products (SOP):

- Expression is the sum (OR) of selected minterms.

| A | B | Minterm (Product) | Decimal Index |
| --- | --- | --- | --- |
| 0 | 0 | A’B’ | m0 |
| 0 | 1 | A’B | m1 |
| 1 | 0 | AB’ | m2 |
| 1 | 1 | AB | m3 |

- Example SOP:

F(A,B)=m1+m3=A’B+ABF(A,B)=m1+m3 = A’B + ABF(A,B)=m1+m3=A’B+AB

Maxterm and POS (Product of Sums):-

Maxterm:

- A sum (OR) term where each variable appears once (either complemented or uncomplemented).

- Example for 2 variables:

  - M0 = A + B

  - M1 = A + B’

  - M2 = A’ + B

  - M3 = A’ + B’

Product of Sums (POS):

- Expression is the product (AND) of selected maxterms.

| A | B | Maxterm (Sum) | Decimal Index |
| --- | --- | --- | --- |
| 0 | 0 | A + B | M0 |
| 0 | 1 | A + B’ | M1 |
| 1 | 0 | A’ + B | M2 |
| 1 | 1 | A’ + B’ | M3 |

- Example POS:

F(A,B)=M0•M2=(A+B)(A’+B)F(A,B) = M0 • M2 = (A+B)(A’+B)F(A,B)=M0•M2=(A+B)(A’+B)