Assignment:- 4A

Binary to Gray code converter using logic gates

Binary → Gray code converter (implementation using logic gates)

1. Introduction

Binary code is the standard positional representation of numbers, whereas Gray code is a code in which two successive numbers differ in only one bit. Converting binary to Gray code is widely used in digital systems, especially in encoders and error reduction.

2. Principle (General n-bit Conversion)

For an n-bit binary input:

- The most significant Gray bit (Gn-1) is the same as the most significant binary bit (Bn-1).

- Gn-1 = Bn-1

- Each remaining Gray bit is the XOR of the corresponding binary bit and the next higher binary bit:

- Gk = Bk+1 XOR Bk   (for k = n-2 down to 0)

3. Boolean Equations (4-bit Example)

For a 4-bit binary input B3 B2 B1 B0, the Gray output bits are:

G3 = B3

G2 = B3 XOR B2

G1 = B2 XOR B1

G0 = B1 XOR B0

If only AND, OR, NOT gates are available, the XOR can be implemented as:

A XOR B = (A AND NOT B) OR (NOT A AND B)

4. Truth Table (4-bit Binary → Gray)

| Binary Input (B3 B2 B1 B0) | Gray Output (G3 G2 G1 G0) |
| --- | --- |
| 0000 | 0000 |
| 0001 | 0001 |
| 0010 | 0011 |
| 0011 | 0010 |
| 0100 | 0110 |
| 0101 | 0111 |
| 0110 | 0101 |
| 0111 | 0100 |
| 1000 | 1100 |
| 1001 | 1101 |
| 1010 | 1111 |
| 1011 | 1110 |
| 1100 | 1010 |
| 1101 | 1011 |
| 1110 | 1001 |
| 1111 | 1000 |

5. Implementation Notes

- Inputs: Binary number B3 B2 B1 B0

- Outputs: Gray code G3 G2 G1 G0

- Logic circuit:

  - G3 is directly connected to B3

  - G2 is XOR of B3 and B2

  - G1 is XOR of B2 and B1

  - G0 is XOR of B1 and B0

Gate requirement for 4-bit converter:

- 3 XOR gates (or equivalent using AND/OR/NOT)

- 1 direct connection

6. Applications

- Digital communication (error minimization)

- Shaft encoders (to avoid spurious outputs)

- Karnaugh map simplification