Study of 8086 Microprocessor Architecture

- Architecture:

The Intel 8086 is a 16-bit microprocessor introduced by Intel in 1978. It is the foundation of the x86 family of processors and was designed to execute complex instructions efficiently.

Main Features:

- 16-bit processor with 20-bit address bus → can access 1 MB memory (2²⁰ bytes)

- Has segmented memory architecture

- Contains Arithmetic Logic Unit (ALU) for arithmetic and logical operations

- 14 registers (general, segment, and pointer/index)

- Clock speed typically between 5–10 MHz

- Supports multiprogramming and pipelining (instruction queue)

Block Diagram:

How it works:

- The BIU fetches the next instruction while EU executes the current one — providing pipelined execution.

- This improves performance by overlapping fetch and execute cycles

- Register Organization:

8086 has several types of registers categorized as:

- General Purpose Registers (GPR):

| Register | Purpose | Description |
| --- | --- | --- |
| AX | Accumulator | Used in arithmetic, logic, and data transfer |
| BX | Base Register | Used in indexed addressing |
| CX | Count Register | Used in loops and shifts |
| DX | Data Register | Used in multiplication/division and I/O operations |

- Segment Registers:

| Register | Purpose |
| --- | --- |
| CS | Code Segment – holds program code |
| DS | Data Segment – holds variables/data |
| SS | Stack Segment – manages stack operations |
| ES | Extra Segment – used for string operations |

- Pointer and Index Registers:

| Register | Purpose |
| --- | --- |
| SP | Stack Pointer |
| BP | Base Pointer |
| SI | Source Index |
| DI | Destination Index |
| IP | Instruction Pointer (points to next instruction) |

- Flag Register:

  - Contains status flags (like Carry, Zero, Sign, Overflow) and control flags.

  - Used to indicate the outcome of operations.

- Memory Segmentation in 8086:

The 8086 uses a segmented memory model to efficiently access its 1 MB address space. Memory is divided into four main segments:

- Code Segment: Contains executable program instructions.

- Data Segment: Holds variables, data, and constants.

- Stack Segment: Stores temporary data, return addresses, and handles function calls.

- Extra Segment: Used for additional data and string operations.

Each segment is 64 KB in size. The physical memory address is calculated using both the segment and offset addresses with the formula:
Physical Address = (Segment × 10H) + Offset

For example, if CS = 2000H and IP = 1234H, then the physical address = 2000H × 10H + 1234H = 21234H.

- Example Programs (Assembly Language)

  - Hello World Program

Code:

section .data

msg db "Hello world!", 0ah

section .text

global _start

_start:

mov rax, 1

mov rdi, 1

mov rsi, msg

mov rdx, 13

syscall

mov rax, 60

mov rdi, 0

syscall

Output:

  - Addition of Two Number

Code:

section .data

num1 db 3

num2 db 5

result db 0

section .text

global _start

_start:

mov al, [num1]

add al, [num2]

add al, '0'

mov [result], al

mov rax, 1

mov rdi, 1

mov rsi, result

mov rdx, 1

syscall

mov rax, 60

xor rdi, rdi

syscall

Output: