DEAM: NUMBER SYSTEM

Binary, Decimal, Octal, Hexadecimal Reference Table:

| Decimal | Binary | Octal | Hexadecimal |
| --- | --- | --- | --- |
| 0 | 0000 | 0 | 0 |
| 1 | 0001 | 1 | 1 |
| 2 | 0010 | 2 | 2 |
| 3 | 0011 | 3 | 3 |
| 4 | 0100 | 4 | 4 |
| 5 | 0101 | 5 | 5 |
| 6 | 0110 | 6 | 6 |
| 7 | 0111 | 7 | 7 |
| 8 | 1000 |  | 8 |
| 9 | 1001 |  | 9 |
|  | 1010 |  | A |
|  | 1011 |  | B |
|  | 1100 |  | C |
|  | 1101 |  | D |
|  | 1110 |  | E |
|  | 1111 |  | F |

Number System Conversions:

- Binary to Decimal:
Method: Multiply each bit with 2^position (right to left), then sum.
Example:
Binary = 1011
= 1×2³ + 0×2² + 1×2¹ + 1×2⁰
= 8 + 0 + 2 + 1 = 11

- Binary to Octal:
Method: Group binary in 3 bits from right to left. Convert each group to octal.
Example:
Binary = 110101
Groups: 000 110 101
Octal = 0 6 5 → 065

- Binary to Hexadecimal:
Method: Group binary in 4 bits from right to left. Convert each group to hexadecimal.
Example:
Binary = 10111100
Groups: 1011 1100
Hex = B C → BC

- Decimal to Binary:
Method: Divide by 2 repeatedly. Collect remainders bottom-up.
Example:
13 ÷ 2 = 6 R1
6 ÷ 2 = 3 R0
3 ÷ 2 = 1 R1
1 ÷ 2 = 0 R1
Binary = 1101

- Decimal to Octal:
Method: Divide by 8 repeatedly. Collect remainders bottom-up.
Example:
65 ÷ 8 = 8 R1
8 ÷ 8 = 1 R0
1 ÷ 8 = 0 R1
Octal = 101

- Decimal to Hexadecimal:
Method: Divide by 16 repeatedly. Convert remainders to hex digits, bottom-up.
Example:
254 ÷ 16 = 15 R14 → E
15 ÷ 16 = 0 R15 → F
Hex = FE

- Octal to Binary:
Method: Convert each octal digit to 3-bit binary.
Example:
Octal = 57
5 → 101
7 → 111
Binary = 101111

- Hexadecimal to Binary:
Method: Convert each hex digit to 4-bit binary.
Example:
Hex = 2F
2 → 0010
F → 1111
Binary = 00101111

- Octal to Decimal:
Method: Multiply each digit by 8^position and sum.
Example:
Octal = 132
= 1×8² + 3×8¹ + 2×8⁰
= 64 + 24 + 2 = 90

- Hexadecimal to Decimal:
Method: Multiply each digit by 16^position and sum.
Example:
Hex = 1A
= 1×16¹ + 10×16⁰
= 16 + 10 = 26

Conversion Procedure Table:

| From \ To | Binary | Decimal | Octal | Hexadecimal |
| --- | --- | --- | --- | --- |
| Binary | - | Multiply by 2^n | Group 3 bits | Group 4 bits |
| Decimal | Divide by 2 | - | Divide by 8 | Divide by 16 |
| Octal | 3-bit binary | Multiply by 8^n | - | Convert via bin |
| Hex | 4-bit binary | Multiply by 16^n | Convert via bin | - |

ASSIGNMENT: 1

Number System Conversion 1’s & 2’s Complement

Q1. Convert binary to decimal

- (110101)₂

= 1×2⁵ + 1×2⁴ + 0×2³ + 1×2² + 0×2¹ + 1×2⁰

= 32 + 16 + 0 + 4 + 0 + 1 = 53

- (101.1101)₂

= 1×2² + 0×2¹ + 1×2⁰ + 1×2⁻¹ + 1×2⁻² + 0×2⁻³ + 1×2⁻⁴

= 4 + 0 + 1 + 0.5 + 0.25 + 0 + 0.0625 = 5.8125

- (10101)₂

= 1×2⁴ + 0×2³ + 1×2² + 0×2¹ + 1×2⁰

= 16 + 0 + 4 + 0 + 1 = 21

- (1011.101)₂

= 1×2³ + 0×2² + 1×2¹ + 1×2⁰ + 1×2⁻¹ + 0×2⁻² + 1×2⁻³

= 8 + 0 + 2 + 1 + 0.5 + 0 + 0.125 = 11.625

- (100101)₂

= 1×2⁵ + 0×2⁴ + 0×2³ + 1×2² + 0×2¹ + 1×2⁰

= 32 + 0 + 0 + 4 + 0 + 1 = 37

Q2. Convert decimal to binary

0   →   BCD: 0000   →   Excess-3: 0011

1   →   BCD: 0001   →   Excess-3: 0100

2   →   BCD: 0010   →   Excess-3: 0101

3   →   BCD: 0011   →   Excess-3: 0110

4   →   BCD: 0100   →   Excess-3: 0111

5   →   BCD: 0101   →   Excess-3: 1000

6   →   BCD: 0110   →   Excess-3: 1001

7   →   BCD: 0111   →   Excess-3: 1010

8   →   BCD: 1000   →   Excess-3: 1011

9   →   BCD: 1001   →   Excess-3: 1100

B0

B1

| 0 | 00 0 | 0 | 0 |
| --- | --- | --- | --- |
| B3 B2 0 | 1 | 1 | 1 |
| X | X | X | X |
| 1 | 1 | X | X |